CNC FLEET DIAGNOSTICS & REAL-TIME DASHBOARD REPORT

Condition Monitoring, Multi-Sensor Anomaly Detection & Live Streaming Simulation | Fleet Units CNC01 – CNC05

1. Executive Summary & Codebase Pipeline

Through comprehensive analysis of the repository's Jupyter notebooks and CSV datasets, we synthesized the analytics pipeline: code2.ipynb performed grouped IQR outlier filtering and degradation health ranking (Monotonicity, Robustness, Trendability) to isolate top predictive features (Temperature, Pressure, Vibration Crest, Current, Vibration P2P) into standardized PCA components (95% variance); code1.ipynb synthesized operational KPIs; and code3.ipynb engineered a 3-way consensus alarm engine (Rule + Z-score + Moving Average with persistence). All algorithms have been unified into a high-performance Python Streamlit dashboard (app.py) featuring live dynamic telemetry streaming.

2. Consolidated Machine Performance & Diagnostic Benchmark Matrix

| Machine | Utilization | Run Hrs | Prod Rate | Total Parts | Scrap % | Avg Temp | Avg Vib | Alarms Detected |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CNC01 | 78.50% | 130.8 h | 72.8 /h | 9,528 | 1.95% | 44.4 °C | 1.05 | 0 (Nominal) |
| CNC02 | 75.50% | 125.8 h | 65.2 /h | 8,210 | 1.96% | 47.3 °C | 1.34 | 0 (High Downtime 25.3h) |
| CNC03 | 77.35% | 128.9 h | 75.5 /h | 9,729 | 2.39% | 46.1 °C | 1.24 | 1 Critical Event (45 min) |
| CNC04 | 78.25% | 130.4 h | 63.5 /h | 8,285 | 2.03% | 50.5 °C | 1.63 | 0 (Thermal/Stress Peak) |
| CNC05 | 77.45% | 129.1 h | 74.8 /h | 9,651 | 1.99% | 44.9 °C | 1.13 | 0 (Nominal Stable) |
| Fleet Total/Avg | 77.41% | 645.1 h | 70.4 /h | 45,403 | 2.07% | 46.7 °C | 1.28 | 1 Confirmed Alarm Event |

3. Implemented Visualization Deliverables (app.py & generate_visualizations.py)

• Operating Status Distribution: Stacked percentage bar chart displaying time proportion across Running (~77.4%), Idle (~12%), Maintenance (~6%), and Fault/Stopped states, plus chronological state transition timelines.

• Temperature & Vibration Trends: High-resolution time-series with 5-sample rolling moving averages, baseline thresholds (μ+2σ), zoom/pan brush controls, and shaded alarm regions.

• Production Counts & Hourly Rates: Stacked output bars (Good parts vs. Scrap) and production rates (parts/running hour) showing CNC03's lead (75.5/h) vs CNC04's longer cycle lag (63.5/h).

• Machine Comparison & Radar Profile: Multi-dimensional normalized radar/spider footprint and benchmark matrix evaluating temperature, vibration, cycle time, motor current, and variability.

• Utilization & Downtime Comparison: Utilization % vs. the 80% industrial target (CNC01 leads at 78.50%) paired with scheduled vs. unscheduled downtime cause analysis (CNC02 highest downtime: 25.33 h).

• Rejection-Rate & Quality Yield: Defect frequency ranking isolating CNC03 as highest risk (2.39% scrap, 233 parts) vs fleet mean (2.07%), cross-correlated with cycle duration.

• Detected Abnormal Periods: Comprehensive event incident log isolating CNC03 sustained anomaly on 2026-01-05 (13:45 to 14:25, 45 min duration, vibration +196% spike to 4.36, temp to 56.5°C, current to 14.8A).

• Alarm Summary & Diagnostics Matrix: Breakdown of individual trigger mechanisms (6 Rule Flags, 16 Z-Score Flags, 17 MA Flags, 14 Consensus Flags) with interactive real-time sensitivity tuning sliders.

4. Real-Time Dynamic Telemetry Simulation Engine

A modular simulation engine was deployed within the dashboard using Streamlit's @st.fragment auto-refresh. Every 1 to 10 seconds (user-configurable), the engine generates dynamic telemetry sampled from machine Gaussian baselines (Temperature, Vibration, Current, Spindle RPM, Pressure, Cycle Time, Output) and executes online consensus anomaly classification. Features include: (1) Live interactive dials/gauges with visual threshold zones, (2) Dynamic rolling 50-point strip charts, (3) 'Inject Fault/Anomaly Spike' button triggering synthetic vibration (+5σ) and thermal (+3.5σ) deviations to verify instantaneous alarm response, and (4) Real-time event log with color-coded operational status badges.

Quick Access:  Launch command: python -m streamlit run app.py (or run_dashboard.bat) | Static visuals & HTML: visualizations/dashboard_report.html